NeuroPipe Design Document

1. Project Overview

NeuroPipe (Neurophysiology Processing and Integration Pipelines for Experiments) is a semi-automated analysis pipeline for EEG data recorded at the Linguistic Research Infrastructure (LiRI) at the University of Zurich.

The goal of NeuroPipe is to provide:

- standardized EEG preprocessing and ERP analysis workflow

- accessible analysis tools for researchers without advanced programming skills

- reproducible analysis pipelines

- training resources for workshops and tutorials

The pipeline will support EEG datasets recorded using the LiRI systems:

- BioSemi ActiveTwo

- Brain Products ActiChamp

The implementation will be based on Python and MNE-Python.

2. Scope of the Pipeline

The NeuroPipe pipeline will integrate the following EEG preprocessing and ERP analysis steps:

- Data import

- Preprocessing

- Artifact handling

- Epoching

- ERP computation

- Visualization

- Quality control

- Export of processed data

The pipeline is designed to be semi-automatic, i.e., at several stages users will be able to (optionally) manually inspect the data.

The pipeline will initially support the following experimental paradigms used in (neuro)linguistics:

- Speech and language processing experiments (perception/production)

3. Target Users

The pipeline is designed for:

- researchers working with LiRI EEG data

- graduate students

- linguistics researchers

- cognitive neuroscience researchers

- workshop participants

Expected user groups include:

- Department of Computational Linguistics, UZH

- Department of Psychology, UZH

- University Psychiatric Hospital Zurich

- NCCR Evolving Language collaborators (various institutions in Switzerland)

4. System Requirements

Language:
Python ≥ 3.10

Core libraries:

- MNE-Python

- NumPy

- SciPy

- Pandas

- Matplotlib

- Seaborn

- Jupyter

Optional:

- MNE-BIDS

- scikit-learn

- autoreject

Environment specification will be provided via: environment.yml

5. Input Data Specification

Supported EEG Systems

The pipeline must support datasets recorded with:

- BioSemi ActiveTwo

- Brain Products ActiChamp

Supported Data Formats

Preferred formats:

- BrainVision (.vhdr/.vmrk/.eeg)

- EDF

- BDF

- MNE FIF

Future compatibility with BIDS EEG format is recommended.

Required Data Components

Each dataset must contain:

- EEG recording file

- Event markers

- Channel information

- Sampling rate

- Experimental condition mapping

Example structure:

dataset/
   sub-01/
      eeg/
         sub-01_task-language_eeg.vhdr
         sub-01_task-language_eeg.eeg
         sub-01_task-language_eeg.vmrk

6. Output Specification

The pipeline should generate the following outputs:

- Processed raw EEG

- Epochs

- ERP averages

- Quality control metrics

- Diagnostic plots

- Log files

Example directory structure:

derivatives/
   sub-01/
      raw_cleaned.fif
      epochs.fif
      evoked_control.fif
      evoked_experimental.fif
      qc_report.html
      figures/
      logs/

7. Pipeline Architecture

The pipeline will be modular. Example modules for NeuroPipe:

- data loading

- preprocessing

- artifact removal

- epoching

- ERP computation

- visualization

- quality control

8. Processing Workflow

Step 1 — Data Import

Example: mne.io.read_raw_brainvision()

Output: Raw MNE object

Step 2 — Channel Setup

Tasks:

- apply electrode montage

- define EEG reference

- rename channels if required

Example:

raw.set_montage("standard_1020")
raw.set_eeg_reference("average")

Step 3 — Filtering

Remove low-frequency drift, high-frequency noise, and electrical interference using bandpass and notch filters.

Example: raw.filter(l_freq=0.1, h_freq=30)

Step 4 — Bad Channel Detection

Identify and reject bad channels with excessive noise using one of the following methods:

- variance threshold

- correlation with neighboring channels

- manual inspection

Bad channels should be stored in:

raw.info["bads"]

Step 5 — Artifact Removal Using ICA

Remove artifacts such as:

- eye blinks (EOG)

- eye movements (EOG)

- muscle activity (EMG)

- optional cardiac artifacts (ECG), if ECG channels are available

Procedure

- Fit ICA to filtered data

- Detect components correlated with EOG channels

- Mark artifact components

- Remove components

Example: ICA(n_components=20, method="fastica")

Step 6 — Event Extraction

Extract events from stimulus markers.

Example: events = mne.find_events(raw)

Step 7 — Epoching

Segment EEG data around event markers.

Example parameters:

tmin = -0.2
tmax = 0.8
baseline = (-0.2, 0)

Example implementation:

epochs = mne.Epochs(raw, events, event_id, tmin, tmax, baseline)

Step 8 — Artifact Rejection

Reject bad epochs with artifacts.

Example threshold: reject = dict(eeg=120e-6)

Implementation: epochs.drop_bad()

Step 9 — ERP Averaging

Compute ERP averages per condition.

Example:

evoked_control = epochs["control"].average()
evoked_experimental = epochs["experimental"].average()

Step 10 — Visualization

Required plots:

- ERP waveform plots

- Topographical maps

- Single-channel time series

Examples:

evoked.plot()
evoked.plot_topomap()

Figures should be saved automatically.

Semi-Automatic User Interaction

The pipeline should include the following checkpoints for manual inspection:

- Inspect raw data

- Confirm bad channels

- Inspect ICA components

- Confirm artifact removal

- Inspect epoch rejection

Implementation options:

- Jupyter Notebook interface

- interactive MNE plots

Configuration

All analysis parameters should be configurable via a YAML configuration file.

Example content of config.yaml:

filter:
  highpass: 0.1
  lowpass: 30

epoch:
  tmin: -0.2
  tmax: 0.8

artifact:
  reject_threshold: 120e-6

Quality Control

The pipeline should generate quality control metrics including:

- number of rejected epochs

- number of bad channels

- number of ICA components removed

- signal-to-noise ratio

QC outputs should be summarized in: qc_report.html

Logging

All pipeline operations should be logged. Log entries should include:

- processing steps

- warnings

- errors

Reproducibility

To ensure reproducibility, the pipeline must include:

- environment specification

- pipeline versioning

- analysis configuration file

Files:

environment.yml
config.yaml
pipeline_version.txt

Testing

Testing will include:

- validation on two example datasets

- functional testing of each module

Datasets will include:

- auditory speech-in-noise experiment

- language processing experiment

Deployment

The NeuroPipe resources will be distributed via LiRI Resource Hub.

Deliverables:

- Jupyter tutorials

- pipeline scripts

- example datasets

- documentation

Future Extensions

Future versions of NeuroPipe may include the following analyses and data modalities:

- time-frequency analysis

- source localization

- group-level statistics

- multimodal pipelines integrating

  - EEG

  - functional near-infrared spectroscopy (fNIRS)

  - electromagnetic articulography (EMA)

  - magnetoencephalography (MEG)